Copperworks American Single Malt Whiskey

Single Cask #7-2 – June 14, 2023

Brew Data

Brews contributing to Cask #7-2

- Brew 1 number: 2016-3-25-1 (FV2)

  - Brew date: 3/25/16

  - Malt Bill:

    - GWM Pale 4000 lbs,

    - Cara 35 605 lbs,

    - Special B 192.5 lbs,

    - Special X 192.5 lbs,

    - Victory 350 lbs,

    - Melanoidin 200 lbs

  - Yeast Source: Elysian Brewing

  - Brewery: Elysian Brewing

  - Volume in fermentation: 1867 gal

  - Original Gravity: 1.07064

  - Terminal Gravity: 1.014

  - Days in fermentation: 19 days at 78 degrees F.

  - Percent ABV at end of fermentation: 7.81

  - Minimum days in cold-conditioning: 22 days

- Brew 2 number: 2016-04-22-1 (FV3)

  - Brew date: 4/22/2016

  - Malt Bill:

    - GWM Pale 4000 lbs,

    - Cara 35 605 lbs,

    - Special B 192.5 lbs,

    - Special X 192.5 lbs,

    - Victory 350 lbs,

    - Melanoidin 200 lbs

  - Yeast Source: Elysian Brewing

  - Brewery: Elysian Brewing

  - Volume in fermentation: 1923 gal

  - Original Gravity: 1.07502

  - Terminal Gravity: 1.0157

  - Days in fermentation: 15 days at 78 degrees F.

  - Percent ABV at end of fermentation: 8.25

  - Minimum days in cold-conditioning: 10 days

Distillation Data

Total # of Wash Distillations:

  - Wash Distillation 1 number: 20160503-1

    - Low Wines Yield: 322.71 gal

    - Low Wines ABV: 21.5

  - Wash Distillation 2 number: 20160510-1

    - Low Wines Yield: 319.45 gal

    - Low Wines ABV: 21.5

  - Wash Distillation 3 number: 20160516-1

    - Low Wines Yield: 333.58 gal

    - Low Wines ABV: 22.1%

Total # of Whiskey Distillations: 3

  - Whiskey Distillation 1 number: 20160505-1

    - New make yield: 113.09 gal

    - New make ABV: 67.8

  - Whiskey Distillation 2 number: 20160512-1

    - New make yield: 114.19 gal

    - New make ABV: 67.75

  - Whiskey Distillation 3 number: 20160518-1

    - New make yield: 106.51 gal

    - New make ABV: 67.8

Maturation Data

- Barreling date:  07/05/2016

- Proofing date for New Make Whiskey: 07/05/2016

- Barrel entry proof: 115 Proof (57.5% ABV)

  - Cooperage: Independent Stave Company – Lebanon, Kentucky

  - First Use: Copperworks Pale Malt – 807 days

  - Barrel fill volume in gallons: 52.56

  - Stave wood: American White Oak

  - Char: #3

- Warehouse conditions: ~70 degrees F.  Average humidity ~50%

- Days of maturing, prior to emptying: 2540 days

- Exit volumes and proof: 36.68 gallons @ 131.2 proof

Bottling Data

- Date Bottled: 6/7/23

- Bottling Proof: 131.2 proof

- Bottles filled for release: 180